A2: Sensor Reading and Display: An Entire Signal Chain

(Electronic Instrumentation- Fall 2024)

- Wheatstone bridge:

- On the left branch, we used two 10k Ohm resistors, to simplify the fraction to 1, which help balancing the bridge

- On the right branch is a LDR and a 22k Ohm potentiometer

- We picked the range of the LDR as 550 - 22k, which is the reason why I chose to use a 22k Ohm potentiometer

- Balanced bridge: R1/R3 = R2/R4

- V difference = Vs . (R2R4+R2-R3R1+R3)

- Amplify the signal:

- To determine the R2 and R1, we calculated the gain

- A = 3,3 / 1,5 = 2.2

- Then once we know the ratio 2R2 / R1 is 1.2, we tried to find a pair of values that match it, it was 1k and 1k5 Ohm.

- On the joining part, we chose 1k Ohm for all the resistors to achieve a gain of 1.

- Ad = Vout / V difference = 1 + 2R2 / R1

- Interface with Raspberry Pi Pico:

- We wrote the code to read the adc, process its scale, and return the corresponding BCD number.

- Display the result:

- I connected the initialized gpio pins to the 7447IC, I disabled the testing function of it (by feeding it with VCC high) and supply 7447IC with 5V from the pico

- Since the 7-segment display is a common anode one, I only need to supply power to one port.

- And instead of using 7 resistors separately to protect the LED, I round it down and put it to the anode, so all of the LED has the same brightness, which is limited through only one resistor.