# High Level Design

The high-level design gives an overview of the robot car as a complete system. It shows the main hardware components that make up the robot car and describes the general capabilities that the car is able to perform. This High Level Design focuses on what the car can do and what parts are responsible for doing it.

Main components:

- The physical frame

It holds and support all the other parts.

- Wheels

Allow the robot car to move in different direction (forward, backward, and sides).

- Motor driver

Controls the Motor based on commands from a controller.

- Power supply

Provides electrical power to the motors.

- Sensors

Prevent the motor car from hitting the walls.

Capabilities of the Robot Car:

The robot car is able to move forward, backwards, and both sides. It also moves from the starting point to the destination point without hitting any walls.

- Driving Forward

- Steering left and right

- Stop safely

This high-level design shows how the robot car is structured and what general functions it can perform, but does not yet include details like wiring or programming, which are covered in the low-level design.

LOW LEVEL DESIGN

DEFINITION

The Low-Level Design (LLD) of the Autonomous Robot Car outlines the specific components, wiring, data flow, and software architecture used to realize the project. This is a more detailed, technical design than the high-level architecture and helps in the actual implementation phase.

1. Physical Frame

- Chassis: Acrylic platform from the Tscinbuny kit

- Assembly:

  - Front: Ultrasonic sensor mounted for obstacle detection

  - Center: ESP32-CAM with adapter board

  - Rear: Motor driver

  - Bottom: Battery pack

  - Motors: DC gear motors attached to side wheels via brackets

- Design Note: Ensure weight distribution is balanced; sensor mount should be fixed and face straight ahead

2. Wheels (Movement Directions)

- Motor Type: 4x geared DC motors

- Control Type: Differential drive (left/right wheel control)

| Movement | Left Side Motors | Right Side Motors |
| --- | --- | --- |
| Forward | Both left motors forward | Both right motors forward |
| Backward | Both left motors backward | Both right motors backward |
| Left Turn | Left motors stopped or backward | Right motors forward |
| Right Turn | Left motors forward | Right motors stopped or backward |
| U-Turn | Left motors forward | Right motors backward |
| Stop | All motors stopped | All motors stopped |

3. Motor Driver

Connections:

- Power Input (VCC): 7.4V Li-ion battery pack

- Motor Outputs:

  - OUT1, OUT2 → Left motor

  - OUT3, OUT4 → Right motor

- Inputs (from ESP32-CAM):

| L298N Pin | ESP32 GPIO |
| --- | --- |
| IN1 | GPIO 14 |
| IN2 | GPIO 12 |
| IN3 | GPIO 13 |
| IN4 | GPIO 15 |
| ENA/ENB | Tied to 5V (or PWM if using speed control) |

4. Power Supply

- Battery: Rechargeable 7.4V Li-ion (2S) from kit

- Regulation:

  - Motors get raw 7.4V

  - ESP32-CAM powered via onboard voltage regulator (5V input via adapter module)

5. ESP32-CAM (Main Controller)

- Functions:

  - Executes motor commands

  - Reads sensor data

  - Streams camera feed (optional)

  - Makes movement decisions

- Programming Language: C++ via Arduino IDE

8. Capabilities Mapped to Hardware/Software

| Capability | LLD Realization |
| --- | --- |
| Drive Forward | digitalWrite(IN1, HIGH); IN2, LOW etc. |
| Backward | Reverse motor pins |
| Turn Left/Right | Opposite polarity to each motor |
| Stop | All motor pins set to LOW |
| Avoid Collisions | Ultrasonic sensor; stop/turn when <15 cm |
| Navigate Maze | Algorithm loop with decision trees (left-hand rule, etc.) |
| Live Camera Feed | ESP32-CAM HTTP stream (IP access) |
| 8. Testing Plan (from Quality Control Section) - Motor test: Forward, backward, left, right, stop - Sensor test: Object placed in front, verify car avoids it - Navigation test: Simple test maze with turn logic - Live feed test: ESP32 IP feed access via browser - Integration: Run full path and verify all HLD requirements |  |