# High Level Design

The high-level design gives an overview of the robot car as a complete system. It shows the main hardware components that make up the robot car and describes the general capabilities that the car is able to perform. This High Level Design focuses on what the car can do and what parts are responsible for doing it.

Main components:

- The physical frame

It holds and support all the other parts.

- Wheels

Allow the robot car to move in different direction (forward, backward, and sides).

- Motor driver

Controls the Motor based on commands from a controller.

- Power supply

Provides electrical power to the motors.

- Sensors

Prevent the motor car from hitting the walls.

- Main Controller - ESP32

Reads data from the ultrasonic sensor and makes decisions to stop the motor if an obstacle is detected.

- Ultrasonic sensor

Detects the distance between the car and any obstacles or wall directly ahead.

- Camera – ESP32-CAM (optional)

Uses simple computer vision to visually detect walls ahead.

Capabilities of the Robot Car:

The robot car is able to move forward, backwards, and both sides. It also moves from the starting point to the destination point without hitting any walls. Continuously monitor the distance in front of the car using the ultrasonic sensor. If the sensor detects an object within a specific range, it sends a signal to the main controller to stop the motor.

- Driving Forward

- Steering left and right

- Ensure the car doesn’t crash into walls

- Constantly checking the environment

- Stop safely

# LOW LEVEL DESIGN

DEFINITION

The Low-Level Design (LLD) of the Autonomous Robot Car outlines the specific components, wiring, and software architecture used to realize the project. This is a more detailed, technical design than the high-level architecture and helps in the actual implementation phase.

1. Physical Frame

- Chassis: Acrylic platform from the Tscinbuny kit

- Assembly:

  - Front: Ultrasonic sensor mounted for obstacle detection

  - Center: ESP32-CAM with adapter board

  - Rear: Motor driver

  - Bottom: Battery pack

  - Motors: DC gear motors attached to side wheels via brackets

- Design Note: Ensure weight distribution is balanced; sensor mount should be fixed and face straight ahead

2. Wheels (Movement Directions)

- Motor Type: 4x geared DC motors

- Control Type: Differential drive (left/right wheel control)

| Movement | Left Side Motors | Right Side Motors |
| --- | --- | --- |
| Forward | Both left motors forward | Both right motors forward |
| Backward | Both left motors backward | Both right motors backward |
| Left Turn | Left motors stopped or backward | Right motors forward |
| Right Turn | Left motors forward | Right motors stopped or backward |
| U-Turn | Left motors forward | Right motors backward |
| Stop | All motors stopped | All motors stopped |

3. Motor Driver

Connections:

- Power Input (VCC): 7.4V Li-ion battery pack

- Motor Outputs:

  - OUT1, OUT2 → Left motor

  - OUT3, OUT4 → Right motor

- Inputs (from ESP32):

| Pico Raspberry Pin | ESP32 GPIO |
| --- | --- |
| IN1 | GPIO 14 |
| IN2 | GPIO 12 |
| IN3 | GPIO 13 |
| IN4 | GPIO 15 |

4. Power Supply

- Battery: Rechargeable 7.4V Li-ion from kit

The 7.4V Li-ion battery powers the motors directly through the motor driver.
The ESP32-CAM receives 5V through its adapter module, which regulates the input voltage from the battery (or external power source).

5. ESP32 - CAM (Main Controller)

Hardware

- ESP32-CAM Module

- Powered via: 5V regulated from adapter

- Communicates over: Wi-Fi to stream video

Functions:

  - Executes motor commands

  - Reads sensor data

  - Streams camera feed (optional)

  - Makes movement decisions

6. Sensors – HC-SR04 Ultrasonic (Obstacle Avoidance)

Hardware

- HC-SR04 Ultrasonic Sensor

- Connected to: Raspberry Pi Pico

Function: Measure distance to wall/obstacle

Mounting: Front-facing, fixed to chassis

Connections:

- VCC → 5V

- GND → GND

- TRIG → GPIO 4

- ECHO → GPIO 5