Robot Car Design Report

1. Overview

This report outlines the design of an autonomous robot car built using components from the Tscinbuny ESP32 Camera Robot Starter Kit. The robot is designed to move independently, detect and avoid obstacles, and optionally stream live video. The report is divided into two sections: High-Level Design and Low-Level Design.

2. High-Level Design

The high-level design explains how the robot works as a complete system. It focuses on the robot’s main functions and the role of each major component.

Main Hardware Components

- Physical Frame

  - The structure that holds all parts together.

- Wheels

  - Let the car move forward, backward, and turn left or right.

- Motor Driver

  - Sends power to the motors based on commands from the controller.

- Power Supply

  - Provides electricity to the motors and the main controller.

- Sensors

  - Detect obstacles to prevent the car from crashing into walls.

- Main Controller – ESP32-CAM

  - Processes sensor input and controls the motors.

- Ultrasonic Sensor

  - Measures the distance to objects in front of the car.

- Camera (Optional)

  - Streams a video feed and can help detect walls using basic vision.

What the Robot Can Do

- Drive forward, backward, left, and right.

- Navigate from a starting point to a destination while avoiding obstacles.

- Continuously check for nearby objects using an ultrasonic sensor.

- Stop automatically if something gets too close.

- (Optional) Stream a live video feed through Wi-Fi.

Main Capabilities Summary

- Drive in multiple directions

- Turn left or right when needed

- Avoid crashing into walls

- Monitor surroundings constantly

- Stop when an obstacle is detected

3. Low-Level Design

The low-level design dives into the details—components, wiring, and software. It explains exactly how everything is connected and how the system is built.

1. Physical Frame

- Chassis: Acrylic frame from the Tscinbuny kit.

- Mounting:

  - Front: Ultrasonic sensor

  - Center: ESP32-CAM with adapter board

  - Rear: Motor driver

  - Bottom: Battery pack

  - Sides: DC motors connected to wheels

Note: Ensure that the sensor is facing straight ahead and that the weight is evenly balanced.

2. Wheels and Movement

- Motors: 4 DC gear motors (2 on each side)

- Drive System: Differential drive (independent control of left/right wheels)

| Movement | Left Motors | Right Motors |
| --- | --- | --- |
| Forward | Forward | Forward |
| Backward | Backward | Backward |
| Turn Left | Stopped/Backward | Forward |
| Turn Right | Forward | Stopped/Backward |
| U-Turn | Forward | Backward |
| Stop | Stopped | Stopped |

3. Motor Driver Wiring

- Power Source: 7.4V Li-ion battery

- Motor Outputs:

  - OUT1 & OUT2 → Left motor

  - OUT3 & OUT4 → Right motor

- Controller Inputs (from ESP32):

| Motor Driver Input | ESP32 GPIO Pin |
| --- | --- |
| IN1 | GPIO 14 |
| IN2 | GPIO 12 |
| IN3 | GPIO 13 |
| IN4 | GPIO 15 |

4. Power Supply

- Battery: 7.4V rechargeable Li-ion battery

- Usage:

  - Directly powers the motors via the motor driver

  - The ESP32-CAM receives a regulated 5V through its adapter module

5. ESP32-CAM (Main Controller)

- Features:

  - Controls motor movements

  - Reads distance data from the ultrasonic sensor

  - Streams camera feed via Wi-Fi (optional)

- Power Input: 5V regulated through adapter

6. Ultrasonic Sensor (Obstacle Avoidance)

- Type: HC-SR04

- Mounted: Front of the robot

- Connected to: Raspberry Pi Pico

| Pin | Connected To |
| --- | --- |
| VCC | 5V |
| GND | Ground |
| TRIG | GPIO 4 (Pico) |
| ECHO | GPIO 5 (Pico) |

- Function: Measures how far an object is from the car. Sends this data to the ESP32 so it can decide whether to stop or turn.

4. Conclusion

This robot car combines simple mechanical parts, microcontrollers, and sensors to build a functional autonomous system. The high-level design outlines what the car can do, while the low-level design provides detailed instructions for building and wiring the robot. The combination of movement, sensing, and optional vision makes this a solid foundation for further robotics experiments and upgrades.